| Subject : 24UBSCS302 - INTRODUCTION TO COMPUTER ARCHITECTURE AND MICROPROCESSOR |  |
| --- | --- |
| Total Mark : 75 | Question Paper ID : 841 |

Section – A 			   	(10 X 2 = 20 Marks)

- What is ASCII code?

- Convert (17)₈ to decimal.

- Define opcode.

- What is the role of the program counter?

- Give the purpose of JMP instruction.

- What is a Stack in 8085?

- Which interrupt has the highest priority in 8085?

- What is the function of RIM instruction?

- What is meant by pipeline hazard?

- State one difference between scalar and vector processing.

- State any two applications of BCD.

- Define 9’s complement and 10’s complement.

Section – B 				(5 X 5 = 25 Marks)

- Convert the following floating-point numbers:
a) (10.625)₁₀ → Binary
b) (101.101)₂ → Decimal

- Perform binary addition using 1’s complement method: (a) 10101 + 01111

- Explain the working of CALL and RET instructions with stack diagram.

- Differentiate between hardware and software interrupts in 8085.

- Explain the concept of instruction pipelining with a neat diagram.

- Write short notes on:
a) BCD Code
b) Gray Code

- Explain the steps involved in converting decimal to BCD with an example.

Section – C 			Answer Any Three Questions 	                   	(3 X 10 = 30 Marks)

- Explain all number systems used in digital computers.

- Draw and explain the architecture of 8085 microprocessor in detail.

- Explain all the Addressing Modes of 8085 with suitable programs.

- Describe Direct Memory Access (DMA) in detail.

- Compare RISC and CISC processors in terms of instruction set, addressing modes, hardware complexity, performance, and applications.